Example 8:

1Kg of H2O at 0 C is heated to 100 C by contact with a heat reservoir at 100 C. What is s for the H2O? What is it for the reservoir?

CPH2O=4.184 KJKg K°

Consider the system to be a constant P process (water may expand).

ΔsH2O=nCPlnT2T1=1 Kg4.184KJKg Kln373173=1.305 KJKg K

large reservoir

Δsres=−dQT2

from 1st Law

Qrev=CPH2OT2−T1

Δsres.=−1.121KJKg⋅K

Ok, process possible

ΔsTotal=0.184KJKg⋅K