Example 4:

N2 reservoir

100 bar

# 300K

2m3

# Large

Cv = 20.9 J/mol K

Tinitial = 280 K

VTotal = 4m3

Pinitial = 1 bar

Step a) Pressurize to 5 bar.

1st Low:

dU=dQ+dW+ehedne

−lhldnl

Process is fast  Q = 0

In step a) W = 0 (V constant)

Also, nothing is leaving & dnl = 0.

The total energy may be written in terms of specific energy:

dU=UdN+NdU

Total internal energy

and he = constant (large reservoir)

Let ne = N and integrate 1st Law equation:

dUhe−U=dNN

Constant V.

A					he−Uinitialhe−U=NNi=PTinitialPinitialT

Defining:	he=CpTe−To+ho

Stand. State

U=CvT−To+Uo

and	ho=Uo+RTo

same as PV

A	becomes:

CpTe−CvTinitialCpTe−CvT=PTinitialPinitialT

or in terms of T:

T=γ Te1+Pinitial/P γ Te/Tinitial−1

abiabatic expansion

Constant V.

here for Te = 300K, Ti = 280K

Pinitial = 1 bar, P = 5 bar,  = 1.4

T=1.4 3001+1/5 1.4300/280−1=382 K

At this time:

N=PVRT=315 moles

Also note: that as P>>Pinitial

T→γTe

independent of the conditions in the tank to begin with!

Step b) The liquid purge is a constant P process (P = 5 bar)

Here:

dU=UdN+NdU

total

specific

specific

dQ=0

−dW=+PdV=PNdV+PVdN

total

specific

specific

∴UdN+NdU=−PNdV−PVdN+hedN

or regroup:

NdU+PdV=NdU+PV=NdH

NdH=he−U−PV−HdN

integrate:

dHhe−H=dNN

yields:

he−Hinitialhe−H=NNinitial=VTinitialVinitialT=Te−TiTe−T

Solving for T:

T=TeVinitial/V Te/Tinitial−1+1

Constant P!

When the tank is drained:

V = 4 m3, Te = 300 K, Tinitial = 382 K

V = 2VI

Part (a)

T=3001/2 300/382−1+1=336 K