LOPA Errata 2/12/2003

Page 128, Equation 7-1 1: Term [SYMBOL FOR INITIATING EVENT FREQUENCY] should be included before x Pignition

Page 128, Scenario lb equation: Term [SYMBOL FOR INITIATING EVENT FREQUENCY] should be included before x Pignition

Page 129, Scenario 2b second equation: the three P terms in brackets after the numerical values should be deleted

Page 146, Table 8.3: Total PFD for all IPLs should be 1 x 10-4  not 1 x 10-5

Page 202, Table A.10: Notes state “(i.e. a total PFD of at least 1 x 10-4 must be in place for IPLs).” Change “1x10-4”  to 1x10-2”.

Page 225, Table B.6: Frequency of Mitigated Consequence should be changed  to “1.21 x 10-6”

Page 250: “Figure F. 1 shows a graph ....... for the scenario in Example F. 2”. Change Example F.2 to Example F.1.