Errata Sheet – January 30, 2006

Guidelines for Chemical Process Quantitative Risk Analysis

Chapter 2:  Consequence Analysis

# American Institute of Chemical Engineers;  New York, NY  2000

## Page

#### Page 68	Table 2.4.  Several table entries are incorrect.

For 180o Standard threaded elbow, K = 0.60 instead of 0.70

For Tees, Run-through, threaded, K = 0.10 instead of 1.10

For Tees, Flanged/welded, K1 = 150 instead of 250; K = 0.50 instead of 0.05

For Check valves, Swing, K = 1.50 instead of 0.0

For check valves, Tilting disk, K = 0.50 instead of 1.50

Bottom number of 0.50 in column under K  should be deleted.

Page 75	Figure 2.4.  Upper and lower k values are reversed, i.e. the lower curve should be

K = 1.2 and the upper curve k = 1.67.

Page 99	Middle of page.  The italicized word Evaporation should be bold Evaporation to be consistent with other headings.

Page 100	Below Equation (2.1.39)

Heat flux should have units of (energy/area-time) instead of (energy/area)

Thermal conductivity should have units of (energy/length-time-deg) instead of (energy/length-deg)

Page 114	Table 2.8.  “E:  Slighrly” should be “E:  Slightly”

Page 121	Figure 2.28.  Scales on y-axis on bottom two figures are off by an order of magnitude.  In both, scale goes from 101 to 104.

Page 122	Table 2.12.

Under urban conditions, for A-B stability class, equation should be

Under urban conditions, for D stability, equation should be

Page 144	Near middle of page.  “demensional” should be “dimensional”

Page 147	Table 2.16.  Heading in first column should be “Concentration Ratio” not “Concentration ration”.

Page 148	Table 2.16.  Heading in first column should be “Concentration Ratio” not “Concentration ration”.

Page 161	Figure 3.3.  The arrival time, impulse and duration should all have units that are divided by (kg TNT)1/3.

Page 170	Table 2.20.  First column.  The words “Planar” and “Line” should be switched.

Page 253	Table 2.29.  “n-butyl acrytate” should be “n-butyl acrylate.”

### Spreadsheets

Example 20:	The impulse, duration and arrival time should be multiplied by (TNT mass)1/3.

Example 21b:	Flame speed at top of spreadsheet has units of Mach no instead of m/s

Excel version:  interpolation for scaled overpressure and scaled impulse is missing.